## Matlab code to do with Robot Arm:

Files structure:

- IP_RobotArm.m   provide IP_RobotArm class and its interface to control the robot.

- IP_Get_Image.m

- Library files *.p  support libraries, must be same folder with IP_RobotArm.m

### IP_Get_Image.m

function IP_RGB_Image = IP_Get_Image(read_src)

Description: dedicated interface to take picture directly from camera. There is no controlling robot. Picture is from current robots pose

Parameter:

- read_src: 'RobotArm'(default)/'Local'

Get image from …

Return: RGB format image with size h:300, w:400

NOTE: please refer to IP_RobotArm.IP_RBA_Take_Picture interface to take workspace picture

Example:

>> image(IP_Get_Image());

### File IP_RobotArm.m

class IP_RobotArm

Description: A class encapsulate Robot Arm control for this course.

Provide two main interfaces: to take workspace image, and grasp object.

- IP_RBA_Take_Picture

- IP_RBA_Grasp_Object_PP

Another auxiliary interfaces:

- IP_RBA_Drop_Object_Bin

- IP_RBA_Drop_Object_Origin

- IP_RBA_Open_Gripper

- IP_RBA_Close_Gripper

- IP_RBA_Go_Home

Parameter:

- debug_m: debug option true(default)/false.

**To run the robot, debug option must be set to false.

Return: Object to manipulate the robot.

Example:

Create a Robot object:

>> arm=IP_RobotArm(false);

class IP_RobotArm

function rtn_image = IP_RBA_Take_Picture(obj)

Description: Robot arm move to certain place to take whole workspace picture.

In this function, robot will do 3 actions:

- Go to default position

- Go to hardcoded place

- Take picture and send its

Parameter: No

Return: Workpace picture in RGB format, with size h:300, w:400

Example:

>> arm=IP_RobotArm(false);

>> img=arm.IP_RBA_Take_Picture();

>> image(img);

class IP_RobotArm

function grasp_result = IP_RBA_Grasp_Object_PP(obj, p1, p2)

Description: Robot arm move to the middle point of p1 and p2 to grasp object.

In this function, robot will do 4 actions:

- Go to default position

- Go to middle point between p1 and p2, and rotates its gripper

- Open gripper and Down to grasp object

- Close gripper and Lip object up

Parameter: two points p1, p2 are pixels in image. They are in format [x y] for x is coordinate along the width of image, y is coordinate along the height of image (See in example).

Two points are no need to be in the image. They can have negative value xy. But their center point must inside the image.

Return: Grasp result True/False.

Example:

>> arm=IP_RobotArm(false);

>> result = arm.IP_RBA_Grasp_Object_PP([71 46],[93 89]);

[INFO]Grasp at:(-118.0,592.0)mm, with angle:{117.10}Deg!

Figure below illustrates grasp point and axes in taken image.

X

Y

Grasp point

**Note: after this command, we can use IP_RBA_Drop_Object_Bin, or IP_RBA_Open_Gripper()command to release grasped object.

>> arm.IP_RBA_Drop_Object_Bin();

## Start system for testing

### Overall computers connection

Robot Computer: 192.168.0.10

Lab Computer: 192.168.0.11

Student Computer: Auto IP

Router

### Robot Computer

Start the Robot control program:

- Execute TM_Inspector.exe in D:\RobotFile

- Select:

- “Robot model” is “Quanta_Robot”

- “Normal”

- Press “Run Rtss” and wait for Robot complete the startup (red turn to green)

- For stop control, press “End Rtss”

Start the robot Server:

- Change feedback data rate to 125 (125ms send once)

- Press “robot Server” to open server

Default Port setting:      IP. 192.168.0.10         port. 6188

125

### Lab Computer

Check Lab computer already connected to router and had IP: 192.168.0.11

Open the Terminal and run command: IP_CameraServer

Camera runs the initialization and starts the Server for image taking as below screenshot.

### Student Computer

Connect to router with Auto IP configuration. (Turn-off wifi)

Start Matlab for control robot and process image.